Firmware change list

Date:            10/07/96

Product:         ServManager

Last revision:   1.9

New revision:    1.C

Flash file name: SVE1-C.HEX

Changes:

1.  Fix port 9-12 CPU type not being initialized correctly

Port 9-12 do not get cpu type(PC, MAC, SUN) initialization parameters

at startup time correctly. Erratic operation will occur on ports 9-12

2.  Add Sun keyboard language type

Support for international keyboard mapping is added.

When a Sun CPU boots up it requests the keyboard type and language

to determine which drivers and character set are to be used.

Former firmware gave a fixed answer of 0, defaulting to a U.S.

keyboard mapping. The language and keyboard type can now be changed.

This is changed from the front panel when Sun keyboard has been

selected. An additional menu item then appears called Sun kb language.

A value from 0-255 can be entered here. All subsequents requests by Sun

computers for the keyboard language type will be given this value.

If the value entered is greater than 128 a keyboard type of 3 will be

sent upon request for keyboard type, otherwise a keyboard type of 4

will be sent. Even though a Sun 5 keyboard type is used these values

are used per Sun specifications. Most installations will use a keyboard

type of 4.

In order for the new language and type to take effect the CPU must be

rebooted after the switch has been configured. The default value is

U.S.(33) which will be set upon reset to factory default. It is not

necessary to reset the unit to factory default. A value of 0 will be

present after upgrading the firmware.

The values corresponding to each language are:

US                 33         Spanish            42

US/UNIX            34         Swedish/Finnish    43

French             35         Swiss-French       44

Danish             36         Swiss-German       45

German             37         United Kingdom     46

Italian            38         Korean             47

Netherlands/Dutch  39         Taiwan             48

Norwegian          40         Japan              49

Portuguese         41         French-Canadian    50

If you do not see your language listed please refer to your Sun manual.

3.  Fix VF display of blank instead of 0 in middle digit

When displaying values of >99 and the middle digit is 0 a space

was displayed instead of a 0. This was annoying but affected the

display only.

4.  Fix Sun release codes not always sent

Sun keyboards issue a release code when all keys that were pressed

have been released. The release code was not always being sent. Most

applications did not care, but some did, primarily affecting the use

of the cursor keys.

5.  Make left Apple key code same as right Apple key code

Mapping from a PC keyboard to the Apple key produced only the

right-hand Apple key code, now it gives the left-hand key code. This

corrects certain Apple applications not recognizing this key when using

a PC keyboard right alt key to map to the Apple key.

6.  Fix Apple extended keyboard II commands from kb don't work

This keyboard and this keyboard alone produces a slight variant of the

keyboard codes. This exception is now checked for. This now will allow

keyboard commands to the switch to work with the Apple extended

keyboard II.